HW#5			Thin Airfoil Theory

- The equation of the illustrated airfoil camber line is as follows:

z

x

c2

c

m

Zc(x)=0                                            0≤x≤ c216mc2x-c2c-x          c2≤x≤c

The maximum camber value is equal to m = 0.04c.

- Using thin airfoil theory, show that the two coefficients A1 and A2 of this airfoil are obtained as follows:

A1=16πmc(π2-1)         A2=-323πmc

- Calculate

The zero-lift angle of attack (αL=0),

The lift coefficient at 3-degree angle of attack.

The moment coefficient about the aerodynamic center.

---------------------------------------------------------------------------------------------------------------------

- The camber profile of an airfoil is given by:

zc=0.2xc-0.25xc2                                            0<x<0.5zc=3401-xc                                                            0.5<x<1

Calculate

a. The zero-lift angle of attack (αL=0),

b. The lift force at an angle of attack of 10 degrees

c. The moment coefficient around the leading edge at an angle of attack of 10 degrees

e. The position of the center of pressure for the value of the coefficient calculated in part b

---------------------------------------------------------------------------------------------------------------------

- The equation of airfoil camber line is as follows:

zc=axx-1x-b

where Zc and x are dimensionless parameters using airfoil chord length (c) and a and b are dimensionless constants.

a. Find an equation for dZcdx(θ) .

b. Find the constant value of b such that Cm ac=0.

Hint: first write an equation for Cm ac in terms of An coefficient and then set it equal to zero.

c. If the value of cl=0.25 and α=0, find the value of a. Also calculate the value of xcp.

Hint:

cos2θ=12+12cos2θ

-----------------------------------------------------------------------------------------------------------

- Consider an airfoil with a 2m chord in a flow with a velocity of 50 m/s at sea level. If the lift per unit span is 1353 N and the airfoil section has a curvature with the following characteristics, what is the angle of attack of this airfoil.

zc=0.250.8xc-xc2                for 0≤xc≤0.4 0.1110.2+0.8xc-xc2   for 0.4≤xc≤1