# Circulation Calculation with Full Integral Steps

# Example Problem:

Consider the velocity field:
V = y^2 i - x^2 j

a) Is the flow rotational or irrotational?
b) Calculate the circulation around the triangle with vertices at (0,0), (1,0), and (1,1) in a counterclockwise direction.

## Part (a): Is the flow rotational or irrotational?

To determine if the flow is rotational or irrotational, we need to compute the vorticity ω = ∇ × V.

Given:
- V_x = y^2
- V_y = -x^2

The vorticity is calculated as:
ω_z = ∂V_y/∂x - ∂V_x/∂y

First, calculate each partial derivative:
∂V_y/∂x = ∂(-x^2)/∂x = -2x
∂V_x/∂y = ∂(y^2)/∂y = 2y

Thus, the vorticity is:
ω_z = -2x - 2y

Since ω_z ≠ 0, the flow is rotational.

. Parameterizing the Triangle

The triangle has three sides. Let's parameterize each side:

- Side 1: From (0, 0) to (1, 0)

  - x = t, y = 0, 0 ≤ t ≤ 1

  - dr = (dx, dy) = (dt, 0)

- Side 2: From (1, 0) to (1, 1)

  - x = 1, y = t, 0 ≤ t ≤ 1

  - dr = (dx, dy) = (0, dt)

- Side 3: From (1, 1) to (0, 0)

  - x = 1 - t, y = 1 - t, 0 ≤ t ≤ 1

  - dr = (dx, dy) = (-dt, -dt)

3. Evaluating the Line Integral

Let's evaluate the line integral over each side and sum them up.

- Side 1:

  - V = (0, -t^2)

  - V • dr = (0, -t^2) • (dt, 0) = 0

  - ∫(Side 1) V • dr = ∫(0 to 1) 0 dt = 0

- Side 2:

  - V = (t^2, -1)

  - V • dr = (t^2, -1) • (0, dt) = -dt

  - ∫(Side 2) V • dr = ∫(0 to 1) -dt = -1

- Side 3:

  - V = ((1-t)^2, -(1-t)^2)

  - V • dr = ((1-t)^2, -(1-t)^2) • (-dt, -dt) = -2(1-t)^2 dt

  - ∫(Side 3) V • dr = ∫(0 to 1) -2(1-t)^2 dt = -2/3

4. Computing the Circulation

Finally, let's sum up the integrals over each side to get the circulation:

Circulation = ∮C V • dr

= ∫(Side 1) V • dr + ∫(Side 2) V • dr + ∫(Side 3) V • dr

= 0 - 1 - 2/3

= -5/3